COLD CHAIN ACTIVE REFRIGERATION (2C TO 8C) & THERMAL BREACH QUARANTINE SOP

ENTERPRISE SERVICE LEVEL AGREEMENT & STANDARD OPERATING PROCEDURE

# 1. Document Header & Scope

| Document ID | QA-SOP-2026-016-COLD |
| --- | --- |
| Effective Date | September 15, 2026 |
| Controlling Department | Global Quality Assurance, Life Sciences Regulatory Compliance, & Cold Chain Logistics |
| Material Classification | Temperature-Sensitive Veterinary Biologics & Pharmaceuticals (MARA-MATKL = VET_MED) |
| Target SAP Tables / Fields | MARA (MATNR, MATKL, TEMPB), LIPS (WERKS, LGORT), LIKP (LIFSK, WBSTK) |

# 2. Commercial Purpose & Operational Context

Establishes strict quality assurance mandates, continuous IoT data logger monitoring, and quarantine procedures for veterinary biologicals, injectable therapeutics, vaccines, and insulin requiring continuous refrigeration between 2.0°C and 8.0°C (35.6°F to 46.4°F). Preserves efficacy and patient safety against thermal degradation.

# 3. Key Definitions & Operational Thresholds

• Active Cold Chain Cargo: Veterinary medical substances marked with SAP temperature condition code '02' requiring active mechanical refrigeration.

• Thermal Excursion: Any temperature reading exceeding 8.0°C or falling below 2.0°C recorded by NIST-calibrated continuous digital IoT data loggers.

• SAP Quarantine Delivery Block 01: A systematic lock placed on SAP LIKP delivery documents preventing commercial release or financial invoice billing until QA audit approval.

# 4. Core Binding SLA / Policy Clauses

4.1 Continuous Active Monitoring
Reefer trailers and thermal containers must carry calibrated IoT cellular temperature sensors transmitting readings every fifteen (15) minutes to the AI Copilot telematics platform.

4.2 Critical Thermal Breach Definition
Any cumulative exposure above 8.0°C for greater than sixty (60) minutes, or any single transient spike above 15.0°C, constitutes an irreversible Critical Thermal Breach.

4.3 Freeze Damage Zero-Tolerance
Because freezing irreversibly denatures biological proteins, any temperature record falling below 0.0°C for more than fifteen (15) minutes requires immediate 100% batch condemnation.

4.4 Mandatory Quarantine Diversion
Upon thermal breach detection, the carrier must immediately halt transit and divert cargo to the nearest certified cold storage depot.

# 5. Financial Matrices, Penalties & Liability Caps

• Carrier Liability for Spoiled Cargo: Carrier liable for 100% of the invoice manufacturing cost of all condemned freight (MARA-NETPR * LIPS-LFIMG).

• Replacement Freight Expedited Cost: Carrier at fault must fund priority air replacement freight up to $1,000.00 USD per clinical order.

• Certified Disposal Fee: Hazardous biological disposal surcharge of $350.00 USD assessed against carrier if reefer unit mechanical failure caused excursion.

# 6. Exceptional Clauses & Relief Criteria

• Pre-Cooling Calibration Grace Period: Transient 15-minute thermal spikes during trailer door opening at loading dock are permissible if core probe remains < 6.0°C.

• Catastrophic Sensor Malfunction: If backup analog chemical phase-change indicator confirms no thaw, electronic false alarms may be overturned by QA Director.

# 7. Autonomous AI Enforcement & System of Record Actions

• IoT sensor reports > 8.0°C for 60 minutes: AI Copilot instantly writes Delivery Block '01' into SAP LIKP, notifies consignee clinic of replacement shipment, and files carrier claim.

• Carrier driver attempts delivery of breached lot: Consignee scanner rejects delivery barcode; goods diverted to QA quarantine storage.